Interdata 16b/32b Simulator Usage

18-Apr-2020

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2008, written by Robert M Supnik

Copyright (c) 1993-2008, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	Interdata Features	3

2.1	CPU (16b)	4

2.2	CPU (32b)	6

2.3	Selector Channel (SELCH0, SELCH1, SELCH2, SELCH3)	8

2.4	Programmed I/O Devices	8

2.4.1	Paper Tape Reader/Punch (PT)	8

2.4.2	Console, Teletype Interface (TT)	9

2.4.3	Console, PASLA Interface (TTP)	10

2.4.4	Line Printer (LPT)	11

2.4.5	Line Frequency Clock (LFC)	12

2.4.6	Programmable Interval Clock (PIC)	12

2.4.7	Floppy Disk Controller (FD)	13

2.4.8	Programmable Asynchronous Line Adapters (PAS, PASL)	13

2.5	Cartridge Disk Controller (DP)	15

2.6	Mass Storage Module/Intelligent Disk Controller (DM)	16

2.7	Magnetic Tape Controller (MT)	17

3	Symbolic Display and Input	18

3.1	16b Instruction Input	18

3.2	32b Instruction Input	19

This memorandum documents the Interdata 16b and 32b simulators.

# Simulator Files

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/interdata/	id_defs.h

id16_cpu.c	[id32_cpu.c]

id16_dboot.c	[id32_dboot.c]

id_dp.c

id_fd.c

id_fp.c

id_idc.c

id_io.c

id_lp.c

id_mt.c

id_pas.c

id_pt.c

id_tt.c

id_ttp.c

id_uvc.c

id16_sys.c	[id32_sys.c]

# Interdata Features

The Interdata simulator includes simulators for a variety of 16b (I3, I4, I5, 70, 80, 7/16, 8/16, 8/16E) and 32b (7/32, 8/32) models.  This is by no means a complete sampling of all the variations in the Interdata/Perkin-Elmer family.  The 32b family included options for special communications instructions (7/32C, 8/32C), as well as a later extension for virtual memory (3200 series).

The Interdata simulator is configured as follows:

device names	simulates

CPU - 16b		Interdata 3, 4, 5, 70, 80, 7/16, or 8/16 CPU with 64KB

memory

Interdata 8/16E CPU with 256KB memory

CPU - 32b		Interdata 7/32 or 8/32 CPU with 1MB memory;

8/32 supports 2 or 8 register banks

SELCH			selector channel (1-4)

PT			paper tape reader/punch

TT			console terminal, Teletype interface

TTP			console terminal, PASLA interface

LFC			line frequency clock

PIC			programmable interval clock

LPT			line printer

FD			floppy disk

DP			2.5MB/10MB cartridge disk with four disk drives

DM			mass storage module (MSM)/intelligent (IDC) disk

controller with four disk drives

MT			magnetic tape

PAS			programmable asynchronous line controller

PASL			programmable asynchronous lines, up to 32

The Interdata simulator implements two unique stop conditions:

- Decode of an undefined instruction, and STOP_INST is set

- Runaway carriage control tape in the line printer

The LOAD command is used to load a carriage control tape for the line printer.  The DUMP command is used to dump a contiguous portion of memory as a self-loading bootstrap paper tape.  The syntax for the DUMP command is:

DUMP <filename> lowaddr-highaddr

The low address must be greater than or equal to X'D0'.

Devices are assigned their default device numbers, as documented in the Interdata literature.  Device numbers can be changed by the command:

SET <device> DEVNO=num

Device number conflicts are not checked until simulation starts.  If there is a device number conflict, simulation stops immediately with an error message.

Selector channel devices are assigned by default to selector channel 0.  Selector channel assignments can be changed by the command:

SET <dev> SELCH=num

Selector channel assignments cannot introduce conflicts.

Most devices can be disabled and enabled, with the commands:

SET <dev> DISABLED

SET <dev> ENABLED

All devices are enabled by default.

## CPU (16b)

The CPU options include memory size and CPU type:

SET CPU I3				Interdata 3 (base instruction set)

SET CPU I4				Interdata 4 (base plus single precision

floating point)

SET CPU 716				Interdata 7/16 (extended instruction set)

(equivalent to Models 5, 70, and 80)

SET CPU 816				Interdata 8/16 (extended plus double

precision floating point)

SET CPU 816E			Interdata 8/16E (extended plus double

precision plus expanded memory)

SET CPU 8K				set memory size = 8KB

SET CPU 16K				set memory size = 16KB

SET CPU 24K				set memory size = 24KB

SET CPU 32K				set memory size = 32KB

SET CPU 48K				set memory size = 48KB

SET CPU 64K				set memory size = 64KB

SET CPU 128K			set memory size = 128KB (8/16E only)

SET CPU 256K			set memory size = 256KB (8/16E only)

SET CPU CONSINT			assert console interrupt (7/16, 8/16,

and 8/16E only)

If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.  Initial memory size is 64KB.

These switches are recognized when examining or depositing in CPU memory:

-a			examine/deposit ASCII characters

-b			examine/deposit bytes

-c			examine/deposit packed ASCII characters

-f			examine/deposit fullwords

-d			data radix is decimal

-o			data radix is octal

-h			data radix is hexadecimal

-m			examine as instruction mnemonics

-v			interpret address as virtual

Packed characters, halfwords, fullwords, and instructions must be aligned on a halfword (16b) boundary.  If an odd address is specified, the low order bit is ignored.

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			16		program counter

R0..R15		16		general registers

FR0..F14		32		single precision floating point registers

D0H..D14H		32		double precision floating point registers,

high order

D0L..D14L		32		double precision floating point registers,

low order

PSW			16		processor status word

CC			4		condition codes, PSW<12:15>

SR			16		switch register

DR			32		display register low 16 bits

DRX			8		display register extension

DRMOD			1		display mode

DRPOS			2		display pointer position

SRPOS			1		switch pointer position

IRQ[0:3]		32		interrupt requests

IEN[0:3]		32		interrupt enables

STOP_INST		1		stop on undefined instruction

STOP_WAIT		1		stop if wait state and no I/O events pending

PCQ[0:63]		16		PC prior to last branch or interrupt;

most recent PC change first

WRU			8		interrupt character

The CPU detects when the simulator is idle.  When idle, the simulator does not use any resources on the host system.  Idle detection is controlled by the SET IDLE and SET NOIDLE commands:

SET CPU IDLE			enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is disabled by default.  The CPU is considered idle if the WAIT STATE flag is set in the PSW.

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 65536 entries.

## CPU (32b)

The CPU options include memory size and CPU type:

SET CPU 732				Interdata 7/32, single precision floating

point

SET CPU DPFP			Interdata 7/32, double precision floating

point

SET CPU 832				Interdata 8/32 (double precision floating

point, 8 general register sets)

SET CPU 2RS				Interdata 8/32 (double precision floating

point, 2 general register sets)

SET CPU 64K				set memory size = 64KB

SET CPU 128K			set memory size = 128KB

SET CPU 256K			set memory size = 256KB

SET CPU 512K			set memory size = 512KB

SET CPU 1M				set memory size = 1024KB

SET CPU CONSINT			assert console interrupt

If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.  Initial memory size is 1024KB.

These switches are recognized when examining or depositing in CPU memory:

-a			examine/deposit ASCII characters

-b			examine/deposit bytes

-c			examine/deposit packed ASCII characters

-w			examine/deposit halfwords

-d			data radix is decimal

-o			data radix is octal

-h			data radix is hexadecimal

-m			examine as instruction mnemonics

-v			interpret address as virtual

Packed characters, halfwords, fullwords, and instructions must be aligned on a halfword (16b) boundary.  If an odd address is specified, the low order bit is ignored.

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			20		program counter

R0..R15		32		active general register set

GREG[32]		32		general register sets, 16 x 2

FR0..FR14		32		single precision floating point registers

D0H..D14H		32		double precision floating point registers,

high order

D0L..D14L		32		double precision floating point registers,

low order

PSW			16		processor status word

CC			4		condition codes, PSW<12:15>

SR			16		switch register

DR			32		display register low 16 bits

DRX			8		display register extension (x/16 only)

DRMOD			1		display mode

DRPOS			2		display pointer position

SRPOS			1		switch pointer position

MACREG[0:15]	32		memory access controller segment registers

MACSTA		5		memory access controller interrupt status

IRQ[0:3]		32		interrupt requests

IEN[0:3]		32		interrupt enables

STOP_INST		1		stop on undefined instruction

STOP_WAIT		1		stop if wait state and no I/O events pending

PCQ[0:63]		20		PC prior to last branch or interrupt;

most recent PC change first

WRU			8		interrupt character

The CPU detects when the simulator is idle.  When idle, the simulator does not use any resources on the host system.  Idle detection is controlled by the SET IDLE and SET NOIDLE commands:

SET CPU IDLE			enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is disabled by default.  The CPU is considered idle if the WAIT STATE flag is set in the PSW.

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 65536 entries.

## Selector Channel (SELCH0, SELCH1, SELCH2, SELCH3)

An Interdata system can have 1 to 4 selector channels (SELCH0, SELCH1, SELCH2, SELCH3).  The default number of channels is 2.  The number of channels can be changed with the command:

SET SELCH CHANNELS=num

All the state for a selector channel can be displayed with the command:

SHOW SELCH num

The selector channels implement these registers:

name			size		comments

SA[0:3]		20		start address, channels 0 to 3

EA[0:3]		20		end address, channels 0 to 3

CMD[0:3]		8		command, channels 0 to 3

DEV[0:3]		8		active device, channels 0 to 3

RDP[0:3]		2		read byte pointer, channels 0 to 3

WDC[0:3]		3		write data counter, channels 0 to 3

IREQ			4		interrupt requests; right to left,

channels 0 to 3

IENB			4		interrupt enables; right to left,

channels 0 to 3

## Programmed I/O Devices

### Paper Tape Reader/Punch (PT)

The paper tape reader and punch (PT units 0 and 1) read data from or write data to disk files.  The RPOS and PPOS registers specify the number of the next data item to be read and written, respectively.  Thus, by changing RPOS or PPOS, the user can backspace or advance these devices.

The paper tape reader supports the BOOT command.  BOOT PTR copies the so-called '50 loader' into memory and starts it running.

The paper tape controller implements these registers:

name			size		comments

RBUF			8		reader buffer

RPOS			32		reader position in the input file

RTIME			24		time from reader start to interrupt

RSTOP_IOE		1		reader stop on I/O error

PBUF			8		punch buffer

PPOS			32		punch position in the output file

PTIME			24		time from punch start to interrupt

PSTOP_IOE		1		punch stop on I/O error

IREQ			1		paper tape interrupt request

IENB			1		paper tape interrupt enable

IARM			1		paper tape interrupt armed

RD			1		paper tape read/write mode

RUN			1		paper tape running

SLEW			1		paper tape reader slew mode

EOF			1		paper tape reader end of file

Error handling is as follows:

type		error	        STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape

in		end of file		1	report error and stop

0	out of tape

in,out	OS I/O error	x	report error and stop

### Console, Teletype Interface (TT)

The Teletype keyboard (TT0) reads from the console keyboard; the Teletype printer (TT1) writes to the simulator console window. The Teletype units (TT0, TT1) can be set to one of four modes, KSR, 7P, 7B, or 8B:

mode	input characters		output characters

KSR	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit set	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

Changing the mode of either unit changes both.  The default mode is KSR.

The Teletype has a BREAK key, which is not present on today's keyboards.  To simulate pressing the break key, stop the simulator and use the command:

SET TT BREAK

Break status will be asserted, and will remain asserted for the interval specified by KTIME.

The Teletype interface implements these registers:

name			size		comments

KBUF			8		input buffer

KPOS			32		number of characters input

KTIME			24		input polling interval (if 0, the keyboard

is polled synchronously with the line clock)

TBUF			8		output buffer

TPOS			32		number of characters output

TTIME			24		time from output start to interrupt

IREQ			1		interrupt request

IENB			1		interrupt enable

IARM			1		interrupt armed

RD			1		read/write mode

FDPX			1		half-duplex

CHP			1		input character pending

### Console, PASLA Interface (TTP)

Later Interdata system connect the system console via the first PASLA interface rather than the Teletype interface.  The PASLA console can be simulated with a Telnet session on the first PAS line.  Alternately, the PASLA console can be attached to the simulator console window, using the TTP device in place of TT.

To switch the simulator console window to TTP, use the command:

SET TTP ENABLED or

SET TT DISABLED

Device TT is automatically disabled and device TTP is enabled.  To switch the simulator console window back to TT, use the command:

SET TT ENABLED or

SET TTP DISABLED

Device TTP is automatically disabled and device TT is enabled.  If TTP is enabled at its default device settings, the base address for the PAS multiplexer must be changed:

SET PAS DEVNO=12

Otherwise, a device number conflict occurs.

The PASLA keyboard (TTP0) reads from the console keyboard; the PALSA printer (TTP1) writes to the simulator console window.  The PASLA units (TTP0, TTP1) can be set to one of four modes, UC, 7P, 7B, or 8B:

mode	input characters		output characters

UC	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit cleared	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

Changing the mode of either unit changes both.  The default mode is 7B.

To simulate pressing the break key, stop the simulator and use the command:

SET TTP BREAK

Break status will be asserted, and will remain asserted for the interval specified by KTIME.

The PASLA console interface implements these registers:

name			size		comments

CMD			16		command register

STA			8		status register

KBUF			8		input buffer

KPOS			32		number of characters input

KTIME			24		input polling interval (if 0, the keyboard

is polled synchronously with the line clock)

KIREQ			1		input interrupt request

KIENB			1		input interrupt enabled

KARM			1		input interrupt armed

CHP			1		input character pending

TBUF			8		output buffer

TPOS			32		number of characters output

TTIME			24		time from output start to interrupt

TIREQ			1		output interrupt request

TIENB			1		output interrupt enable

TIARM			1		output interrupt armed

### Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.  The default position after ATTACH is to position at the end of an existing file. A new file can be created if you attach with the -N switch.

In addition, the line printer can be programmed with a carriage control tape.  The LOAD command loads a new carriage control tape:

LOAD <file>				load carriage control tape file

The format of a carriage control tape consists of multiple lines.  Each line contains an optional repeat count, enclosed in parentheses, optionally followed by a series of column numbers separated by commas.  Column numbers must be between 0 and 7; column seven is by convention top of form.  The following are all legal carriage control specifications:

<blank line>			no punch

(5)					5 lines with no punches

1,5,7					columns 1, 5, 7 punched

(10)2					10 lines with column 2 punched

0					column 0 punched

The default form is 1 line long, with all columns punched.

The line printer implements these registers:

name			size		comments

BUF			7		last data item processed

BPTR			8		line buffer pointer

LBUF[0:131]		7		line buffer

VFUP			8		vertical forms unit pointer

VFUL			8		vertical forms unit length

VFUT[0:131]		8		vertical forms unit table

IREQ			1		line printer interrupt request

IENB			1		line printer interrupt enable

IARM			1		line printer interrupt armed

POS			32		position in the output file

CTIME			24		character processing time

STIME			24		spacing operation time

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

### Line Frequency Clock (LFC)

The line frequency clock (LFC) frequency can be adjusted as follows:

SET LFC 60HZ			set frequency to 60Hz

SET LFC 50HZ			set frequency to 50Hz

The default is 60Hz.

The line frequency clock implements these registers:

name			size		comments

IREQ			1		clock interrupt request

IENB			1		clock interrupt enable

IARM			1		clock interrupt armed

TIME			24		clock frequency

The line frequency clock autocalibrates; the clock interval is adjusted up or down so that the clock tracks actual elapsed time.

### Programmable Interval Clock (PIC)

The programmable interval clock (PIC) implements these registers:

name			size		comments

BUF			16		output buffer

RIC			16		reset interval and rate

CIC			12		current interval

DECR			10		current decrement value

RDP			1		read byte select

OVF			1		interval overflow flag

IREQ			1		clock interrupt request

IENB			1		clock interrupt enable

IARM			1		clock interrupt armed

If the interval requested is an exact multiple of 1 millisecond, the programmable clock auto-calibrates; if not, it counts instructions.

### Floppy Disk Controller (FD)

Floppy disk options include the ability to make units write enabled or write locked.

SET FDn LOCKED			set unit n write locked

SET FDn WRITEENABLED		set unit n write enabled

Units can also be set ENABLED or DISABLED.

The floppy disk supports the BOOT command.  BOOT FDn copies an autoload sequence into memory and starts it running.

The floppy disk controller implements these registers:

name			size		comments

CMD			8		command

STA			8		status

BUF			8		buffer

LRN			16		logical record number

ESTA[0:5]		8		extended status bytes

DBUF[0:127]		8		transfer buffer

DBPTR			8		transfer buffer pointer

IREQ			1		interrupt request

IENB			1		interrupt enabled

IARM			1		interrupt armed

CTIME			24		command response time

STIME			24		seek time, per cylinder

XTIME			24		transfer time, per byte

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

Floppy disk data is buffered in memory; therefore, end of file and OS I/O errors cannot occur.

### Programmable Asynchronous Line Adapters (PAS, PASL)

The Programmable Asynchronous Line Adapters (PAS and PASL) represent, indistinguishably, individual PASLA interfaces, 2 line asynchronous multiplexers, and 8 line asynchronous multiplexers, with a maximum of 32 lines.  All the lines are modeled as a terminal multiplexer, with PAS as the multiplexer controller, and PASL as the individual lines. The PASLAs perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH PAS <port>			set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.

Each line (each unit of PASL) can be set to one of four modes, UC, 7P, 7B, or 8B:

mode	input characters		output characters

UC	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit cleared	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

Each line (each unit of PASL) can also be set for modem control with the command SET PASLn DATASET.  The defaults are 7b mode and DATASET disabled.  Finally, each line supports output logging.  The SET PASLn LOG command enables logging on a line:

SET PASLn LOG=filename		log output of line n to filename

The SET PASLn NOLOG command disables logging and closes the open log file, if any.

Once PAS is attached and the simulator is running, the terminals listen for connections on the specified port.  They assume that the incoming connections are Telnet connections.  The connections remain open until disconnected either by the Telnet client, a SET PAS DISCONNECT command, or a DETACH PAS command.

Other special PASLA commands:

SHOW PAS CONNECTIONS 		show current connections

SHOW PAS STATISTICS 		show statistics for active connections

SET PASLn DISCONNECT		disconnects the specified line.

The controller (PAS) implements these registers:

name			size	comments

STA[0:31]		8		status, lines 0 to 31

CMD[0:31]		16		command, lines 0 to 31

RBUF[0:31]		8		receive buffer, lines 0 to 31

XBUF[0:31]		8		transmit buffer, lines 0 to 31

RIREQ			32		receive interrupt requests;

right to left, lines 0 to 31

RIENB			32		receive interrupt enables

RARM[0:31]		1		receive interrupt armed

XIREQ			32		transmit interrupt requests;

right to left, lines 0 to 31

XIENB			32		transmit interrupt enables

XARM[0:31]		1		transmit interrupt armed

RCHP[0:31]		1		receiver character present, lines 0 to 31

The lines (PASL) implements these registers:

name			size		comments

TIME[0:31]		24		transmit time, lines 0 to 31

The additional terminals do not support save and restore.  All open connections are lost when the simulator shuts down or PAS is detached.

## Cartridge Disk Controller (DP)

Cartridge disk options include the ability to make units write enabled or write locked, and to select the type of drive:

SET DPn LOCKED			set unit n write locked

SET DPn WRITEENABLED		set unit n write enabled

SET DPn 2315			set unit n to 2315 (2.5MB)

SET DPn 5440			set unit n to 5440 (10MB)

Units can also be set ENABLED or DISABLED.

The cartridge disk supports the BOOT command.  To boot OS16/32, the hex form of the operating system file's extension must be placed in locations 7E:7F.  The disk bootstrap looks for a valid OS16/32 volume descriptor in block 0, and uses that to locate the volume directory.  It then searches the directory for a filename of the form OS16xxxx.hhh or OS32xxxx.hhh, where the xxxx is ignored and hhh is the ASCII form of the extension from locations 7E:7F.  The 32b bootstrap can also boot Wollongong UNIX; locations 7E:7F must be 0.  The bootstrap normally boots from the first (removable) platter in a 5440; to boot from the second (fixed) platter, use BOOT -F.

All drives have 256 8b bytes per sector.  The other disk parameters are:

drive cylinders surfaces sectors

2315	   203	  2	   24

5440	   408	  4	   12

The cartridge disk controller implements these registers:

name			size		comments

CMD			3		current command

STA			8		controller status

BUF			8		controller buffer

HDSC			8		current head/sector select

CYL			8		current cylinder select

DBUF[0:255]		8		transfer buffer

DBPTR			16		transfer buffer point

DBLNT			16		transfer buffer length

FIRST			1		first DMA service flag

IREQ			5		interrupt requests; right-to-left,

controller, drives 0 to 3

IENB			5		interrupt enables

IARM[0:3]		1		interrupts armed, drives 0 to 3

STIME			24		seek latency, per cylinder

RTIME			24		rotational latency, per sector

WTIME			24		inter-word latency

Error handling is as follows:

error			processed as

not attached	disk not ready

end of file		assume rest of disk is zero

OS I/O error	report error and stop

## Mass Storage Module/Intelligent Disk Controller (DM)

MSM/IDC disk controller options include the ability to make units write enabled or write locked, and to select the type of drive:

SET DMn LOCKED			set unit n write locked

SET DMn WRITEENABLED		set unit n write enabled

SET DMn MSM80			set unit n to storage module, 80MB

(67MB formatted)

SET DMn MSM300			set unit n to storage module, 300MB

(262MB formatted)

SET DMn MCCD16			set unit n to medium capacity, 16MB

(13.5MB formatted)

SET DMn MCCD48			set unit n to medium capacity, 48MB

(40.5MB formatted)

SET DMn MCCD80			set unit n to medium capacity, 80MB

(67MB formatted)

SET DMn MSM330F			set unit n to storage module, 330MB

(300MB formatted)

Units can also be set ENABLED or DISABLED.

The MSM/IDC controller supports the BOOT command.  To boot OS16/32, the hex form of the operating system file's extension must be placed in locations 7E:7F.  The disk bootstrap looks for a valid OS16/32 volume descriptor in block 0, and uses that to locate the volume directory.  It then searches the directory for a filename of the form OS16xxxx.hhh or OS32xxxx.hhh, where the xxxx is ignored and hhh is the ASCII form of the extension from locations 7E:7F.  The 32b bootstrap can also boot Wollongong UNIX; locations 7E:7F must be 0.  Note that only the MSM80 and MSM300 drives can be bootstrapped; the boot code does not recognize the other drives.

All drives have 256 8b bytes per sector.  The other disk parameters are:

Drive	 cylinders surfaces sectors

MSM80	     823	  5	   64

MSM300     823	 19	   64

MCCD16     823	  1	   64

MCCD48     823	  3	   64

MCCD80     823	  5	   64

MSM300F   1024	 16	   64

The MSM/IDC disk controller implements these registers:

name			size		comments

STA			8		controller status

BUF			8		controller buffer

SEC			8		current sector select

DBUF[0:767]		8		transfer buffer

DBPTR			16		transfer buffer point

DBLNT			16		transfer buffer length

FIRST			1		first DMA service flag

CWDPTR		2		controller write data byte pointer

DWDPTR		1		drive write data byte pointer

IREQ			5		interrupt requests; right-to-left,

controller, drives 0 to 3

IENB			5		interrupt enables

SIREQ			5		saved interrupt requests

ICARM			1		controller interrupt armed

IDARM[0:3]		1		drive interrupts armed, drives 0 to 3

STIME			24		seek latency, per cylinder

RTIME			24		rotational latency, per sector

WTIME			24		inter-word latency

Error handling is as follows:

error			processed as

not attached	disk not ready

end of file		assume rest of disk is zero

OS I/O error	report error and stop

## Magnetic Tape Controller (MT)

Magnetic tape options include the ability to make units write enabled or write locked.

SET MTn LOCKED			set unit n write locked

SET MTn WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET MTn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW MTn CAPAC			show unit n capacity in MB

Units can also be set ENABLED or DISABLED.

The magnetic tape supports the BOOT command.  BOOT MTn copies an autoload sequence into memory and starts it running.

The magnetic tape controller implements these registers:

name			size		comments

CMD			8		command

STA			8		status

BUF			8		buffer

DBUF[0:65535]	8		transfer buffer

DBPTR			16		transfer buffer pointer

DBLNT			16		transfer buffer length

XFR			1		transfer in progress flag

FIRST			1		first DMA service flag

IREQ			4		interrupt requests; right to left,

drives 0 to 3

IENB			4		interrupt enables

IARM[0:3]		1		interrupts armed, drives 0 to 3

STOP_IOE		1		stop on I/O error

WTIME			1		word transfer time

RTIME			1		interrecord latency

UST[0:3]		8		unit status, drives 0 to 3

POS[0:3]		32		tape position, drives 0 to 3

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		set error flag

OS I/O error	set error flag; if STOP_IOE, stop

# Symbolic Display and Input

The Interdata simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display byte as ASCII character

-c			display halfword as two packed ASCII characters

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII character

" or -c		two packed ASCII characters

alphabetic		instruction mnemonic

numeric		hexadecimal number

## 16b Instruction Input

Instruction input uses standard Interdata assembler syntax.  There are seven instruction classes: short branch, extended short branch, short immediate, register, register-register, memory, and register-memory.

Short branch instructions have the format

sbop mask,address

where the mask is a hex (decimal) number between 0 and F (15), and the address is within +32 (forward branch) or -32 (backward branch) of the current location.

Extended short branch instructions have the format

sbxop address

where the address is within +32 or -32 of the current location.  For extended short branches, the simulator chooses the forward or backward direction automatically.

Short immediate instructions have the format

siop regnum,immed

where the register number is a hex (decimal) number, optionally preceded by R, between 0 and F (15), and the immediate is a hex digit between 0 and F.

Register instructions have the format

rop regnum

where the register number is a hex (decimal) number, optionally preceded by R, between 0 and F (15).

Register-register instructions have the format

rrop regnum,regnum

where the register numbers are hex (decimal) numbers, optionally preceded by R, between 0 and F (15).

Memory instructions have the format

mop address{(index)}

where address is a hex number between 0 and 0xFFFF, and the index register is a hex (decimal) number, optionally preceded by R, between 1 and F (15).

Register-memory instructions have the format

rmop regnum,address{(index)}

where the register number is a hex (decimal) number, optionally preceded by R, between 0 and F (15), the address is a hex number between 0 and 0xFFFF, and the index register is a hex (decimal) number, optionally preceded by R, between 1 and F (15).

## 32b Instruction Input

Instruction input uses standard Interdata assembler syntax.  There are nine instruction classes: short branch, extended short branch, short immediate, 16b immediate, 32b immediate, register, register-register, memory, and register-memory.  Addresses, where required, can be specified as either absolute numbers or relative to the current location (.+n or .-n).

Short branch instructions have the format

sbop mask,address

where the mask is a hex (decimal) number between 0 and F (15), and the address is within +32 (forward branch) or -32 (backward branch) of the current location.

Extended short branch instructions have the format

sbxop address

where the address is within +32 or -32 of the current location.  For extended short branches, the simulator chooses the forward or backward direction automatically.

Short immediate instructions have the format

siop regnum,immed

where the register number is a hex (decimal) number, optionally preceded by R, between 0 and F (15), and the immediate is a hex digit between 0 and F.

16b immediate instructions have the format

i16op regnum,immed16{(index)}

where the register number is a hex (decimal) number, optionally preceded by R, between 0 and F (15), the immediate is a hex number between 0 and 0xFFFF, and the index register is a hex (decimal) number, optionally preceded by R, between 1 and F (15).

32b immediate instructions have the format

i32op regnum,immed32{(index)}

where the register number is a hex (decimal) number, optionally preceded by R, between 0 and F (15), the immediate is a hex number between 0 and 0xFFFFFFFF, and the index register is a hex (decimal) number, optionally preceded by R, between 1 and F (15).

Register instructions have the format

rop regnum

where the register number is a hex (decimal) number, optionally preceded by R, between 0 and F (15).

Register-register instructions have the format

rrop regnum,regnum

where the register numbers are hex (decimal) numbers, optionally preceded by R, between 0 and F (15).

Memory instructions have the format

mop address{(index)} or

mop address{(index1,index2)}

where address is a hex number between 0 and 0xFFFF, and the index registers are hex (decimal) numbers, optionally preceded by R, between 1 and F (15).

Register-memory instructions have the format

rmop regnum,address{(index)} or

rmop regnum,address{(index1,index2)}

where the register number is a hex (decimal) number, optionally preceded by R, between 0 and F (15), the address is a hex number between 0 and 0xFFFF, and the index registers are hex (decimal) numbers, optionally preceded by R, between 1 and F (15).

For memory operands, the simulator automatically chooses the format (RX1, RX2, RX3) that consumes the fewest bytes.  If both RX1 and RX2 are feasible, the simulator chooses RX1.